This is my first contribution to the HP48 forum on CIS. My comms. surcharges

are rather high where I am so I can't upload any larger items at present. I

produced this short doc. while experimenting on the HP48 and it is rather

incomplete (lots of ?'s) but it may be of interest nonetheless, especially the

last part (3.), which can be used to dump the HP48 ROM and so forth directly

to the serial port!

Anthony Berkow [100274,621@compuserve.com]                   7/26/93

-----------------------------------------------------------------------------

HP-48SX Special ATTN Commands and Self-Test Environments

========================================================

The ATTN (ON) key is used to get the HP48's attention (see HP 48SX Owner's

Manual, vol. I, pg. 27, 54).

1. ATTN Commands

Various multiple key sequences involving the ATTN key activate special commands,

including the self-test environment and memory view/edit environment (see HP

48SX Owner's Manual, vol. I, pg. 101-102; vol. II, pg. 605, 665-672; HP 48

Programmer's Reference Manual, pg. 415). All the following ATTN commands are

issued as follows:

a. Press and hold ATTN (ON)

b. Press and release the indicated key

c. Release ATTN

KEY     COMMAND

A F     Memory Reset. (Both keys A and F must be held down for this command.)

(see HP 48SX Owner's Manual, vol. I, pg. 101-102; vol. II, pg. 666)

B       ? [seems to do something, but what?]

C       System Halt. Causes a jump to address #00000 (warmstart). ? Any library

configuration objects (programs) found at this time will be evaluated

(executed). ? (See HP 48SX Owner's Manual, vol. II, pg. 666.)

D       Activates the HP48's special self-test environment (the memory view/edit

environment is also activated from here). The HP48 will beep and three

vertical lines will appear on the display to indicate self-test mode has

been activated. Once in this mode, various single keypresses activate

specific commands as described further on. The various ATTN commands

will all still operate when in this mode. To exit the self-test

environment, issue a System Halt command. (See HP 48SX Owner's Manual,

vol. II, pg. 667-672.)

E       Automatic Self-Test. Performs various tests (SPD, DISP, IROM, IRAM) in

sequence, automatically, in a continuous cycle. (Can also be activated

from the self-test environment by pressing Y.) Test results are output

to the serial port. (See HP 48SX Owner's Manual, vol. II, pg. 667.)

PRINT   (G) (MTH) Print Display. (See HP 48SX Owner's Manual, vol. II, pg. 605.)

4       ? [seems to do something, but what?]

SPC     Coma Mode. Turns off the HP48, putting it into "Coma mode" (very low

power with the system clock stopped). When ON is pressed, the Warmstart

Log is cleared and the system warmstarts. (See HP 48 Programmer's

Reference Manual, pg. 415.)

+       Increase Contrast. Increases the display contrast by one step each time

+ is pressed. (See HP 48SX Owner's Manual, vol. II, pg. 665.)

-       Decrease Contrast. Decreases the display contrast by one step each time

- is pressed.

2. Self-Test Environment

The HP48's special self-test environment is activated by pressing ATTN D as

described above. The HP48 will beep and three vertical lines will appear on the

display (left, centre and right) to indicate self-test mode has been activated.

Once in this mode, various commands can be activated by pressing the indicated

keys. As each command is issued, the HP48 will beep, the display is cleared, a

short message indicating the test is displayed in the top-left corner of the

display, and the test follows. During the test, various 5-digit hexadecimal

error codes may be displayed and output to the serial port. At the end of the

test the HP48 will beep, usually indicate OK, FAIL and/or an error code, and

send the test results to the serial port, a single vertical line then

reappearing down the centre of the display. The various ATTN commands listed

above can still be used within this environment. To exit the self-test

environment (including various repeating tests), issue a System Halt command

(ATTN C). (See HP 48SX Owner's Manual, vol. II, pg. 667-672.)

KEY     COMMAND

A       Speed Test. SPD is displayed, followed by what could be the operating

speed of the HP48 ? (usually about 1950000). ?

B       Display Test. DISP is displayed. Press any key repeatedly to step

through six different test patterns, eventually ending the test.

C       Internal 256KB ROM Test. IROM is displayed, followed after a short delay

by the test result (usually OK).

D       Internal 32KB RAM Test. IRAM is displayed, followed after a short delay

by the test result (usually OK).

E       Keyboard Test, Full. KBD1 is displayed. All 49 keys are tested. Press

each key in turn from A through to +, row by row. A beep will be heard

as each key is pressed. If an incorrect key is pressed, an error code

will be generated where digit 1 is 0, digits 2 and 3 indicate the code

of the key pressed (from 01h through to 31h), and digits 4 and 5

indicate the code of the key expected. At the end of the test, KBD OK or

KBD FAIL will be indicated. (See HP 48SX Owner's Manual, vol. II, pg.

667-668.)

F       Keyboard Test, Quick. KBD2 is displayed. Nine keys are tested, covering

all rows and columns of the keyboard. Press each of five keys in turn

from B through to <- in a diagonal line, followed by each of four keys

in turn from ALPHA through to SPC in a diagonal line. If an incorrect

key is pressed, an error code will be generated as for KBD1 above. At

the end of the test, KBD OK or KBD FAIL will be indicated.

G       ESD Test. ESD 210V  210V is displayed, and the HP48 beeps continuosly

at about 5.4 beeps per second, until any key is pressed. ? (ESD OK or

ESD FAIL should apparently be displayed at some stage. ?)

H       (IO) Serial Loop-Back Test. U_LB is displayed. Pins 2 and 3 of the

serial connector (TX and RX) should be carefully shorted before this

test is run. OK is usually displayed after a short delay, otherwise an

error code is generated. (See HP 48SX Owner's Manual, vol. II, pg.

671-672.)

I       UART (Universal Asynchronous Receiver/Transmitter) Test. UART is

displayed, and the test completes after a few seconds, followed by the

test result. ?

J       Port Test. PORT is displayed, followed by a four digit number (usually

1011), which could be a binary number representing the peripherals

connected to the CPU. ?

K       (LIBRARY) Port RAM Test. RAM1 is displayed, followed by an error code if

port 1 does not contain a card, is a ROM card, or is a RAM card with the

read/write switch set to the read only position. Otherwise, the size of

the RAM memory (in bytes) is displayed on the right-hand side of the

display, and after a short delay (memory test) an OK message should be

displayed. The test is then repeated for RAM2. (See HP 48SX Owner's

Manual, vol. II, pg. 669-670.)

L       ? [seems to do something, but what?]

M       Send Test. SEND is displayed, followed after a short delay by the test

result. Probably a Kermit protocol test. ?

N       Receive Test. RECV is displayed, the HP48 then waiting for something

indefinitely. Probably a Kermit protocol test. ?

O       IR Loop-Back Test. IRLB is displayed. The IR filter should be in place

before this test is run. OK is usually displayed after a short delay,

otherwise an error code is generated. (See HP 48SX Owner's Manual, vol.

II, pg. 670-671.)

P       IR I/O Test. IRIO is displayed, and the test completes after a few

seconds, followed by the test result. Possibly an inter-HP48 IR Kermit

protocol test. ?

S       Display Start. Start: is displayed, followed by a 13 digit hexadecimal

number that appears to be some time value measured in system ticks.

Press any key to return to self-test environment. ?

T       Display Fail. Fail : is displayed, followed by a 13 digit hexadecimal

number that appears to be some time value measured in system ticks.

Press any key to return to self-test environment. ?

U       Automatic Self-Test. Performs various tests (SPD, DISP, IROM, IRAM,

RAMp, etc.) in sequence, automatically, in a continuous cycle. Test

results are output to the serial port. ?

V       Automatic Self-Test. Performs various tests (SPD, DISP, IROM, IRAM,

U_LB, etc.) in sequence, automatically, in a continuous cycle. Test

results are output to the serial port. ?

W       Automatic Self-Test. Performs various tests (SPD, DISP, IROM, IRAM,

U_LB, RAMp, etc.) in sequence, automatically, in a continuous cycle.

Test results are output to the serial port. ?

X       Automatic Self-Test. Performs various tests (SPD, DISP, IROM, IRAM,

IRLB) in sequence, automatically, in a continuous cycle. Test results

are output to the serial port.

Y       Automatic Self-Test. Performs various tests (SPD, DISP, IROM, IRAM) in

sequence, automatically, in a continuous cycle. (Can also be activated

at any time by pressing ATTN E as shown above.) Test results are output

to the serial port. (See HP 48SX Owner's Manual, vol. II, pg. 667.)

Z       Automatic Self-Test. Performs various tests (SPD, DISP, IROM, IRAM,

IRLB, RAMp, etc.) in sequence, automatically, in a continuous cycle.

Test results are output to the serial port. ?

DEL     Test Status. Seems to display either OK-48 if no test failures have

occured, or FAIL followed by an error code if any tests have failed. ?

ENTER   Time Init is displayed until any key is pressed. ?

<-      Activates the HP48's special memory view/edit environment. The HP48 will

beep and a hexadecimal address will be displayed, followed by the 16

nibbles of data stored at that address onwards. Once in this mode, the

various keys act as described further on. The various ATTN commands will

all still operate when in this mode. To exit the memory view/edit

environment, issue a System Halt command.

3. Memory View/Edit Environment

The HP48's special memory view/edit environment is activated by pressing ATTN D

followed by <- as described above. The HP48 will beep and a hexadecimal address

will be displayed in the top line of the display, followed by the 16 nibbles of

data stored at that address onwards. Once in this mode, the various keys act as

described below. The various ATTN commands listed above can still be used within

this environment. To exit the memory view/edit environment, returning to the

self-test environment, press any key not specifically listed below, or issue a

System Halt command (ATTN C).

The first address to be displayed (usually #705D9) is always the address of a

routine that displays the version number and copyright message of the HP48

within this environment. The program can be executed by pressing EVAL as

described below.

KEY     FUNCTION

+       Increments address by 1.

-       Decrements address by 1.

x       (TIMES)  Increments address by 100h.

/       (DIVIDE) Decrements address by 100h.

^       (UP CURSOR) (K)   Increments address by 1000h.

v       (DOWN CURSOR) (Q) Decrements address by 1000h.

X       Sets address to current display memory area DA2 (Stack Display) (usually

in the region of #F0B00).

ENTER   Sets address to device I/O area =_IODevices (#00100). (USE WITH

CAUTION!)

Y       Sets address to WSlog in internal 32KB RAM (#F000A). (See HP 48

Programmer's Reference Manual, pg. 415.)

Z       Sets address to 128KB port RAM1 =_RAM1 (#80000).

DEL     Sets address to 128KB port RAM2 =_RAM2 (#C0000).

.       Outputs currently displayed address and contents (16 nibbles) to the

serial port (in ASCII hex.). The address is then incremented by 16.

SPC     Outputs currently displayed address and contents (16 nibbles) to the

serial port (in ASCII hex.). The address is then incremented by 16.

This operation is automatically repeated until 10000h nibbles (4096

lines) have been output. (This operation may be cancelled by issuing a

System Halt (ATTN C).)

EVAL    Executes the routine at the current address. (Performs a SYSEVAL for the

currently displayed address.) Pressing EVAL at the entry address

(usually #705D9) displays the HP48's version and copyright message.

Pressing EVAL at address #00000 results in a System Halt (ATTN C).

0-9,A-F Pressing any of these hexadecimal values causes that nibble to be

written to memory at the current address. The address is then

incremented by 1.