These programs embody a new way of performing infrared remote control

sampling on the HP-48.  The previously distributed remote sampling

programs have sampled IR input by continually reading the IR input and

immediately writing it out to memory.  These programs use a different

approach--they continually sample the IR input while incrementing a

timing count, and only write information to memory when the IR state

changes or the counter overflows.  This approach increases the effective

sampling rate under many circumstances and greatly reduces the amount

of memory needed to store samples.  I have also paid special attention

to instruction timing to make sure that the timing characteristics of

the sampling and playback routines are as nearly identical as possible;

this has been a problem with some previous attempts at remote sampling.

One of my motivations for writing these programs was that the other

IR samplers I tried wouldn't work at all with my VCR.  Both of these

work with my VCR, although I have obtained mixed results using these

programs with other kinds of remotes.  I am hoping that these programs

will be more effective than previous attempts, and that the techniques

I have used here can be improved upon by others.

There are two separate IR sampling systems included here.  The first,

in irsamp.star, irplay.star, ir.asc, and ir.uue, were the first programs

I developed that worked.  They store 3-nibble sample units--1 nibble for

the current IR input state, and 2 nibbles for a repeat count.  The

inner loop of the sampling and playback routines runs at about 25

microseconds per loop iteration, and take about 40 microseconds to

write out the sample data and re-enter the inner loop.  These routines

will make samples of about 90 bytes that work with my VCR.  The second,

in irsamp2.star, irplay2.star, ir2.asc, and ir2.uue, use a similar

approach but instead have only a 1-nibble repeat count.  The inner loop

of these routines runs at about 23 microseconds per iteration, but

sample string are a little over twice as long, at 224 bytes for samples

that work with my VCR.  Interestingly enough, though, Lutz Vieweg's

RFP program will pack samples created by either set of routines to about

the same length of 60 bytes.

This posting includes STAR assembler source for all machine-language

routines and ASC and uuencoded directories containing the machine-

language programs and user RPL wrapping functions that perform additional

type checking and provide a cleaner interface to the provided routines.

The directory IR contains the 3-nibble sampling routines, and the directory

IR2 contains the 2-nibble sampling routines.  IR contains the user RPL

programs SAMPLE and PLAYBACK and the machine code objects IRSAMP and IRPLAY.

IR2 contains SAMPLE2, PLAYBACK2, IRSAMP2, and IRPLAY2.  Similarly, irsamp.star

and irplay.star are the STAR source for the 3-nibble sampling/playback

routines, while irsamp2.star and irplay2.star are the source for the 2-nibble

routines.

The program SAMPLE (SAMPLE2) will take either a real number in level

1, which is used to generate a string of that length to receive the sample,

or a string which will be overwritten with sample data.  It calls the

machine-language program IRSAMP (IRSAMP2) which performs the sampling.

IRSAMP checks for a string object in level 1, which must have SIZE > 1, and

aborts if it doesn't find one.  It then waits up to 15 seconds for IR input,

(that is, for you to place your remote control next to the calculator and

press the button for the remote function you want to sample) and aborts if

none is received in that time.  Otherwise it begins sampling and exits when

the end of the string in level 1 is reached.  I have found that holding an

IR remote too close to the calculator during sampling will often result in

ineffective samples.

The program PLAYBACK (PLAYBACK2) will take a string in level 1 and feed

it to IRPLAY (IRPLAY2), then drop the string.  You can feed strings

directly to IRPLAY if you wish, but they will be left on the stack

(although this is a useful way to test samples without having to recover

them with LASTARG).  You should only feed strings created with IRSAMP to

PLAYBACK or IRPLAY.  It is possible to create strings which leave the

HP-48 IR LED on for long periods of time, which drains the batteries and

may cause hardware damage.  In any case, IRPLAY will turn the IR LED off

before exiting.

Two of my friends who have tested these programs have reported that they

have experienced crashes while using them, although one report did not

appear to involve memory corruption or any other damage.  However, I

do not guarantee these programs to be bug-free--they may cause crashes,

memory corruption, or even hardware damage if used improperly, although

I can also say that I have had no problems for over a week since I

debugged the routines in the IR directory.  Neither are they guaranteed

to work with any IR remote control, although I suspect they will have

greater success than previous remote sampling programs.

I would like to thank Dave Marsh for his original IR sampling programs,

which I have copied small parts of and which were my main source of

technical information in writing these programs, and Jan Brittenson for

his excellent STAR assembler and MLDL debugger which made writing and

debugging these programs easy.